	[image: image7.png]TANGRAM




	Product Specification
	Date of Release:
	2021/4/16

	
	
	Version:
	V1.08



TGM-CME0

	[image: image1.png]



	Features
· Dual Core Processor is powerful for ende computing
· Rich wireless communication interface incl. BT, WiFi, NB-IoT/Cat-M1 for industrial application

· RS-232/485 interface is easy for external device expand

· Backup battery for emergency situations


Specifications
	System
	Processor
	ESP32-WROOM-32D

32-bit LX6 Dual Core 240MHz

	
	Operation System
	RTOS

	LED Indicators
	Power
	x1 Red

	
	WLAN/BT
	x1 Green for WiFi and Yellow for BT

	
	WWAN
	x1 Green

	Sensor
	On Board
	x1 Vibration, x1 Temp., x1 dB Meter *1

	Interface
	General
	x1 RS-232/485 *2

	
	Card
	x1 MicroSD & MicroSIM Combo

	Wireless
Communication
	WLAN
	802.11 b/g/n 2.4GHz

	
	Bluetooth
	V4.2 EDR and BLE

	
	WWAN
	LTE-FDD: *4

B3(1800MHz), B8(900MHz)

	GNSS
	Satellite
	GPS, GLONASS, BeiDou, Galileo

	Antenna
	External
	x3 RPSMA Female

	Power System
	Voltage Input
	12V, 1A

	
	Backup Battery
	3.6V 1150mAh Li-Ion Battery *3

	Mechanical
	Mount
	Screw

	
	Dimension
	84mm x 85mm x 30mm

	
	Weight
	210g

	Environment
	Operating Temp.
	-10 ~ 50OC

	
	Storage Temp.
	-20 ~ 60OC

	
	IP Rating
	IP54

	
	Shock/Vibration
	SAE J1455

	
	Certification
	NCC, BSMI


Dimensions
[image: image2.png]



[image: image3.png]7450





[image: image4.png]



Interface Layout
[image: image5.png]


  [image: image6.png]



· Antenna: 
WiFi/BT (Left), GNSS (Middle), CAT-NB/M (Right)
· I/O Door: 
MicroSD and MicroSIM, 
DIP Switch for System ON/OFF & Charger ON/OFF

· M8:

Power In and RS-232/485

Page 1 of 2

[image: image7.png]